Legal Eagle XL Wing Rib Notes (using Iron Designs rib & drawings)
Starting at the wing root. Ribs #1, #2, #3, & #5 are basic ribs (unmodified gussets) with the only modifications being a double 1/4" vertical at the front of ribs #1 & #2, and on both front & back of the rear spar opening (for the wing fitting notches). You can use two, ¼” x ¼” strips or a single ½” x ¼” for these double size verticals. 
Ribs #4 & #6 are basic ribs with modified gussets (on root side only) for compression strut clearance. I drew lines showing the compression struts on my drawing so I could cut the gussets to the correct size.
Rib #7 is highly modified behind the rear spar (basic in front of rear spar). It uses a K-block & K-block gussets, plus the doublers to support the aileron hinge block. Remember these two ribs are not identical. They are mirror images of each other since one is for the left wing and the other for the right. They have the doublers on opposite sides.

Ribs #8 & #10 are built like ribs #4 & #6 in front of the rear spar (compression strut gussets) but they have K-blocks & K-block gussets added in the rear.

Ribs #9 & #11 are basic ribs in front of the rear spar, but K-blocks & K-block gussets are added in the rear.

Rib #12 is a basic rib in front of the rear spar but has doublers for the aileron hinge block support and a K-block added in the rear. The doublers cover the K-block. Remember these two ribs are not identical. They are mirror images of each other since one is for the left wing and the other for the right. They have the doublers on opposite sides.
Rib #13 is a basic rib in front of the rear spar but has an added doubler (root side only) to support the outer side of the aileron hinge block. Remember these two ribs are not identical. They are mirror images of each other since one is for the left wing and the other for the right. They have the doublers on opposite sides. No K-block on this one.
Ribs #7 & #12 have one K-block added to the aileron side of where the built-in K-block is hidden between the aileron hinge block support doublers. These K-blocks support the ends of the aluminum cover for the aileron cove. Don't worry about adding them or how they fit while you are building the ribs. I won't add mine until I am assembling the ribs to the spars on the build table.

I hope this helps when building your ribs.

Keith

